FlexNet-Dokumentation           ETHEREMU.EXE      (C)1995 Tom Sailer HB9JNX

Stand: 12.6.95

Ident: ETHEREMU

Ethernet Packet Driver Emulator fuer PC/FlexNet und User-FlexNet

Dieser Emulator ist noch BETA! Bitte Bugreports an den Autor (Adresse unten-

stehend), NICHT!!! an DK7WJ. Benutzung auf eingenes Risiko!

Jeder, der diese Software einsetzt, ist verpflichtet, einen Bugreport (auch

im Erfolgsfall) mit Hardwarebeschreibung, Beschreibung der verwendeten

Software (bitte inkl. Betriebssystem/Version, residente Software, FlexNet-

Versionen usw.) an den Autor zu schicken (Packet-Mail bzw. Internet-Mail

bevorzugt.)

Dieser Treiber emuliert nach oben hin einen Ethernet Packet Driver, nach

unten hin ist es eine PC/FlexNet-Applikation. Es erlaubt, TCP/IP ueber

AX25 mit Standard PC Applikationen wie NCSA Telnet zu betreiben. Der Treiber

emuliert Ethernet-ARP und unterstuetzt AX25-ARP (ungetestet).

Installation

------------

Zuerst muss PC/FlexNet mit den L1-Treibern installiert und gestartet werden.

Danach wird ETHEREMU gestartet. ETHEREMU braucht als Argument das eigene

Rufzeichen+SSID, das fuer TCP/IP benutzt werden soll (es darf nicht durch

andere, auf demselben PC/FlexNet aufsetzende Applikationen benutzt werden!)

und die INT-Nummer, wo es das Packet Driver Interface installieren soll.

Diese INT-Nummer darf noch nicht belegt sein!

ETHEREMU -i=INT -m=MYCALL

Beispiel: ETHEREMU -i=0x70 -m=HB9JNX-11

Nun muessen die Routen zu den Routern konfiguriert werden. Dies geschieht mit

EEMUCFG. Es koennen (derzeit) maximal 16 Routen konfiguriert werden.

Ein Beispiel: Es soll eine Route zu HB9W-11 ueber HB9W definiert werden.

HB9W-11 hat die IP-Addresse 44.142.52.6. Es sollen alle Amateur-IP-Addressen

(44.x.x.x) ueber HB9W-11 geschickt werden.

EEMUCFG add vc 44.142.52.6/0xff000000 HB9W-11 HB9W

Das vc weist ETHEREMU an, eine AX25-Verbindung zum Router aufzubauen. Dies

ist die empfohlene Methode. Die Alternative waere dg, womit ETHEREMU die

Packete als UI sendet. Die Hexadezimal-Zahl nach dem Slash ("/") ist die

Netzmaske. Nur diejenigen Address-Bits werden verglichen, deren zugehoeriges

Bit in der Netzmaske 1 ist. Die Konfiguration wird zweckmaessigerweise mit

einem Batch-File erledigt, da ETHEREMU die Konfiguration nicht auf der Platte

abspeichert.

Mit EEMUCFG und EEMUCFG STAT kann nun die Tabelle mit den Pfaden zu den

Routern eingesehen werden. Die eigene IP-Adresse kann nicht direkt konfiguriert

werden. Sie wird fuer AX25-ARP benoetigt und wird gesetzt, sobald die auf

diesen Treiber aufsetzende Applikation den ersten Ethernet-ARP Request taetigt.

Diese IP-Adresse wird also von der Applikation uebernommen.

EEMUCFG STAT zeigt die Zeit an, die seit der letzten Benutzung dieses Eintrags

vergangen ist, und die AX25 Roundtrip Zeit (Frack, in 100ms) an. "--" bedeutet,

dass es sich entweder um eine "datagram" Verbindung handelt, oder dass gerade

kein connect zum Ziel besteht. Connects werden nach 15min Leerlauf getrennt.

Konfiguration der Applikation

-----------------------------

Bei der Applikation muessen zwei Dinge eingestellt werden:

- Die MTU (Maximum Transmission Unit) MUSS!! auf 250 eingestellt werden.

Dies ist die maximale Packetgroesse, die gesendet werden kann. Ethernet

kann hier 1508 Bytes, Packet Radio nur 256. Packete groesser als 256 Bytes

werden von ETHEREMU kommentarlos geloescht!

- Alle Verbindungen sollten ueber einen der konfigurierten Router gehen

Wie das genau geht, haengt von der Applikation ab.

Getestete Applikationen

-----------------------

KA9Q-Derivate

-------------

Funktioniert problemlos.

NCSA Telnet

-----------

Funktioniert soweit. Beim Aufstarten erscheint die Meldung "network jammed",

diese ruehrt daher, dass AX25 viel langsamer als Ethernet ist und der Packet

Driver die Notbremse zieht und eine Fehlermeldung zurueckgibt. Diese Meldung

kann ignoriert werden.

---------------------------------------------------------------------------

FlexNet documentation           ETHEREMU.EXE      (C)1995 Tom Sailer HB9JNX

Date: 12.6.95

Ident: ETHEREMU

Ethernet packet driver emulator for PC/FlexNet and User-FlexNet

This driver is still BETA! Please direct bug reports to the author (see

below), NOT!!! to DK7WJ. Use at your own risk!

Everyone who uses this software is obliged to send a bug report (even in the

case of success) to the author. The report should include a description of

the hardware used, the software (operating system, resident programs, and

their versions!). The preferred way is via packet-mail or internet-mail.

This driver emulates an Ethernet packet driver on one side, and is a

PC/FlexNet application on the other side. It enables standard TCP/IP

applications such as NCSA Telnet to be used over radio. The driver

emulates Ethernet-ARP and supports AX25-ARP (untested).

Installation

------------

First, PC/FlexNet and its L1 drivers must be installed and started. Then,

ETHEREMU may be loaded. ETHEREMU needs the callsign and SSID that should be

used for TCP/IP, and the interrupt number where it should install the packet

driver interface. The interrupt must not be used already by another program!

The callsign and SSID must not be used by another PC/FlexNet application

on the same PC/FlexNet AX25 stack.

ETHEREMU -i=INT -m=MYCALL

Example: ETHEREMU -i=0x70 -m=HB9JNX-11

Now the paths to the routers must be configured. This can be done with

EEMUCFG. There may be 16 paths (at this time).

An example: We want to enter the path to HB9W-11, which can be found over

HB9W and has the IP address 44.142.52.6. All amateur ip addresses (44.x.x.x)

shall be routed to HB9W-11.

EEMUCFG add vc 44.142.52.6/0xff000000 HB9W-11 HB9W

The vc tells ETHEREMU to use virtual circuits, i.e. it connects first to

the router before it sends data. This is the recommended setting. The

alternative would be dg. In this case ETHEREMU uses Unnumbered Info (UI)

frames. The hexadecimal number after the slash ("/") specifies the netmask.

Only those address bits whose corresponding bit in the netmask is one are

compared. For convenience the configuration should be done with a batch file,

since ETHEREMU does not remember the configuration.

EEMUCFG and EEMUCFG STAT shows the table of the paths. The proper IP address

cannot be set directly. It is needed for AX25 ARP replies and it is set the

first time the IP application sends an Ethernet-ARP request. This IP address

is thus taken from the IP application.

EMUCFG STAT shows the time the table entry was last used and it shows the

round trip time of the route (Frack, in 100ms units). A "--" means that it

is either a "datagram" path or that there is currently no circuit. Curcuits

are disconnected if they are not used for 15 minutes.

Configuring the application

---------------------------

Two things must be tuned in the application:

- The MTU (Maximum Transmission Unit) must be set to 250. This is the maximum

packet size the application sends. Ethernet may transport up to 1508 bytes,

while virtually all AX25 implementations are limited to 256 bytes. Packets

bigger than 256 bytes are thrown away by ETHEREMU without notice!

- All outgoing packets must be configured to go through one of the configured

routers.

It is highly application dependent how this can be accomplished.

Tested applications

-------------------

KA9Q derivatives

----------------

They work flawlessly.

NCSA Telnet

-----------

It works. Upon startup, the message "network jammed" appiers. This is due

to the slowness of AX25 compared to Ethernet. ETHEREMU tries to brake the

application by returning an error message when its transmission queue is full.

This message can be ignored.

Autor:  Tom Sailer

Weinbergstrasse 76

CH-8408 Winterthur

Packet: HB9JNX @ HB9W.CHE.EU

EMail:  sailer@ips.id.ethz.ch